# Roof Sheathing Installation Guide (2024 IRC)

Contractor Field Reference Sheet

Applicable to Residential Construction – Verify Local Amendments

## 1. Materials

• APA-rated OSB or Plywood

• Install with strength axis perpendicular to rafters/trusses

• Panels must be grade-stamped and approved for roof use

## 2. Minimum Sheathing Thickness (Typical Prescriptive)

• 16" OC framing: Minimum 7/16" OSB or 3/8" plywood

• 24" OC framing: Minimum 15/32" or 1/2" panels

## 3. Fastening Requirements

• Fasteners: 8d common nails (2 1/2" x 0.131") OR 8d ring-shank nails

• Field nailing: 12" on center

• Edge nailing: 6" on center

• High wind zones may require 4" edge spacing

• Avoid over-driven or under-driven nails

## 4. Panel Installation

• Maintain 1/8" spacing between panels

• Stagger end joints between rows

• Provide edge support using blocking or approved H-clips where required

## 5. Pre-Inspection Checklist

☐ Panels installed perpendicular to framing

☐ 1/8" spacing provided at panel edges

☐ Proper nail size and spacing used

☐ Edge support installed where required

☐ No damaged, delaminated, or rotten sheathing

☐ Perimeter nailing meets wind design requirements

Note: Always confirm local building department amendments and approved construction documents.